Parameter Identification of a DC Motor Based on Moments and Pasek’s Developed Methods

M. Hadef *,  A. Bourouina*,  M.R. Mekideche**

*Département d’Electrotechnique, Université de Constantine  25000–Algérie

**Université de Jijel, Laboratoire LAMEL, B.P. 98, Ouled Aissa 18000 Jijel–Algérie

Email : hadef_mounir@yahoo.fr

Abstract––Pasek described a testing procedure for determining  the dc parameters from the current response to a step in the armature voltage motor. Time moments have been introduced in automatic  control because of the analogy between the impulse response of a linear system and a probability function. This basic idea has generated applications in identification, model order reduction and controller design. In this paper, two developed identification algorithms based on the moments  and Pasek’s methods, are introduced and applied to the parameter identification of a dc motor. The simulation and experimental results are presented and compared.

Index  Terms––Identification, moments method , Pasek’s identification method, dc motor.

- INTRODUCTION

THE DC MOTOR is the obvious proving ground for advanced control algorithm in electric drives due to the stable and straight forward characteristics associated with it[1], [2]. It is also ideally suited for trajectory control applications. The requirement for high performance speed control of dc motors  has produced great research efforts in the application of modern control theory[1], [2], [4]. System identification is the subject which deals with the problem of building  mathematical models of dynamical systems based on the observed system data[2]. The actual identification of models from data involves decision making on the part of the person in search of models, as well as fairly demanding computations to furnish bases for these decisions. There are situations when identification is necessary even though a relatively accurate mathematical model is available. For example, the dc motor parameters might be subject to some time variations, in the cases, a mathematical model that is accurate at the time of the design may not accurate at a later time. In the literature there are many classical methods to identify theses parameters[1]-[6].

Time moments have been introduced in automatic control because of the analogy between the impulse response of a linear system and a probability function[6],[7]. Thus, an impulse response is characterized by an infinity of moments, practically, only the first ones are necessary as for a probability density function. This basic idea has generated applications in identification, model order reduction and controller design, known as the method of moments.

Pasek described a testing procedure for determining the parameters of a dc motor linear model from the current response to a step in the armature voltage of the motor. The Pasek’s equation for the parameter determination has been derived in his work [8].

This paper is structured as follows. Section II, describes  the dynamic of the separately excited dc motor. Section III, describes a new identification algorithm for dc motor parameters by Pasek’s technique. Section IV, describes the method of moments and analyzes the developed identification algorithm, applied to the parameter identification of a dc motor. In section V, experimental results are illustrated and compared  to the simulation results. Finally, conclusions of the paper are summarized in section VI.

- DC MOTOR MODEL

The dynamic of the separately excited dc motor may be expressed by the following equations

Kωt=−Raiat−Ladiatdt+Uat             (1)

Kiat=Jdωtdt+fωt+TLt                (2)

where K, Ra , La , J  and f are respectively, the torque and back-EMF constant, the armature resistance, the armature inductance, the rotor mass moment of inertia and the  viscous friction coefficient. ω(t), ia(t), Ua(t) and TL(t) respectively denote the rotor angular speed, the armature current, the terminal voltage and the load torque.

- DEVELOPED MODEL BASED ON THE PASEK METHOD

Pasek proposed a simple method, which requires one test for the identification of the dynamic model of the separately excited dc motor, with the assumption : f=0 ( viscous friction coefficient null)[8]. Accordingly we thought of developing a model of identification based on the Pasek model, by introducing the viscous friction coefficient.

It is possible from a terminal voltage step ΔUa to determine the majority of the dc motor parameters, that is also possible from the abrupt terminal voltage variation. We measure the initial and final currents and speeds values shown on table III. At the steady state we can write

Ua0=Raia0+Kω0                               (3)

Ua1=Raia1+Kω1                                (4)

Where Ua0, ia0 and ω0 Respectively denote the terminal voltage armature current and the rotor speed at initial regime subscripted "0". Ua1, ia1 and ω1 Respectively denote the terminal voltage, armature current and the rotor speed at final regime subscripted "1".

According to (3) and (4) the torque and back-EMF constant K  can be written as

K=Ua1−ia1ia0Ua0ω1−ia1ia0ω0                                 (5)

the steady state check

ΔUa=RaΔia+KΔω                           (6)

where  ΔUa, Δia  and Δω respectively denote the terminal voltage variation, armature current variation and rotor speed variation.

The armature resistance is given from (6) as

Ra=ΔUa−KΔωΔia                                (7)

according to equations (8) and (9) we can obtain the two transfer functions of the armature current and rotor speed

ΔUa=RaΔia+Ladiadt+KΔω                          (8)

KΔia=JdΔωdt+fΔω                              (9)

the armature current transfer function is given as

H1s=ΔiasΔUas=fK2+Raf1+Jsf1+τmτes2+τm+μτes               (10)

the rotor speed transfer function is given as

H2s=ΔωsΔUas=KK2+Raf1+τmτes2+τm+μτes            (11)

Where

τe=LaRa                    Electrical time constant

τm=RaJK2+Raf        Mechanical time constant

μ=RafK2+Raf         Usually small coefficient

Let us pose

λ=τmτe=Ra2JLa(K2+Raf)                        (12)

for a terminal voltage step ΔUa  the current form is given as

Δia(t)=JΔUaK2+Raf[1T2−T1e−tT2−e−tT1+fJ(1+1T2−T1.T1e−tT1−T2e−tT2)] (13)

Where

T1,2=2λτeλ+μ−1±a         are  the transfer function poles

With                          a=1−4λ(λ+μ)2

We determine the moment t1 when the current pass by its maximum, noted  Δia(t1) we have

t1=ln1+a−2μ1−a+2μλ+μλτea                            (14)

By dividing  the two terms of equation (14) by τe we find

t1τe=λλ+μln(1+a−2μ1−a+2μ)a                      (15)

Let us pose

δ=Δia(2t1)Δia(t1)                                     (16)

Then

δ=λΔUaaRa(λ+μ)L1+fΔUaK2+Raf(1+L2)λΔUaaRa(λ+μ)L3+fΔUaK2+Raf(1+L4)            (17)

Where L1, L2, L3 and L4 are shown on the appendix.

From two equations (15) and (17) we deduce the abacuses(Fig. 7) which gives δ and t1τeaccording to λ.

Measurements of t1, Δia(t1) and Δia(2t1) define the dc motor parameters thanks to the following step

- We calculate δ and an abacus gives λ

- The other abacus gives t1τe

- We deduce τe and  λ τe = τm

- Finally from τe and  λ τe we deduce respectively La and J

the calculation of  output current  gain gives

Ki=Δia=fΔUaK2+Raf                             (18)

from  (18) we calculate

f=K2KiΔUa−RaKi                             (19)

The static torque can be calculated from steady state as

Tst=Kia0−fω0                                (20)

- METHOD OF MOMENTS

The moments constitute the basis for a non classical representation of linear systems. The characterization of an impulse response by its moments is equivalent to the moment characterization of a probability density function[7]. Impulse response moments are system invariants. Like for a probability density function, it is not necessary to compute an infinity of moments to characterize with a good approximation the shape of the impulse response only the first ones are necessary to perform this characterization.

## A.  Temporal Moment of a Function

Let us consider a stable linear system, characterized by its impulse h(t) then,

Hs=BsAs                                        (21)

H(s) can be expanded in Taylor series in the vicinity of s=jω0

Hs=n=0∞−1ns−jω0nAn,ω0                  (22)

where

An,ω0=0∞tnn!ht e−jω0tdt

is the n th order frequency moment of h(t) for ω=ω0, notice that An,ω0is complex. In the particular case  ω0=0,  frequency moments correspond to classical time moments

Anh=0+∞tnn!ht dt                                     (23)

they permit the characterization of H(jω) around ω0=0, as well as that of the impulse response h(t). A0(h) is the area of h(t),  A1(h) defines mean time of h(t) and A2(h) deals with the dispersion of h(t) around its mean time[7]. Equation (22) is rewritten as

Hs=n=0∞−1nsnAnh                               (24)

Let  Hs=n=0∞Snn!dnHsdsns=0

Then, time moments can be expressed as

Anh=−1nn!dnHsdsns=0                          (25)

## B.  Moments and Parameters of a Transfer Function

Let y(t) the step response of the studied system. We proposes to identify the system by the model

Hs=YsEs=K1.1+b1s+b2s2+.....+bmsm1+a1s+a2s2+.....+ansn             (26)

from the final value theorem, as time approaches infinity for a stable linear system, the system response approaches a steady state value K1 given by

K1=limt→∞yt=y                           (27)

if a step input is applied to the system described in equation (26), by taking the Laplace transform of the normalized response gives

Hs=s.ys                                        (28)

let us consider ε(t) an error function with

εt=K1−yt                                  (29)

by introducing the Laplace transform in equation (29), (26) can be written as

εs=K11−1s.1+b1s+b2s2+.....+bmsm1+a1s+a2s2+.....+ansn             (30)

the development of (30) gives

εs=K1a1−b1+.....am−bmsm−1....ansn−11+a1s+a2s2+..............+ansn        (31)

Then, using (24) we can write

εs=n=0∞−1nsnAnε                        (32)

according to (24) and (32) we can deduce the coefficients of the transfer function H(s) by solving the following matrix system

K1a1−b1K1a2−b2.....K1an+1−bn+1=A0ε0.....0−A1εA0ε......0........................................Anε.........1a1.....an

(33)

where An(ε) is the nth  order temporal moment.

## C.  Dc Motor Transfer Function  and  its Moments

For our cases, when n=2 and m=1, the transfer function (8) becomes

Hs=K1.1+b1s1+a1s+a2s2                            (34)

system (33) is reduced to the following matrix system

K1a1−b1K1a20=A000−A1A00A2−A1A0.1a1a2                    (35)

the resolution of this matrix system (35), gives the following coefficients:

Δia(2t1)

ia1

ia0

Δia(t1)

t1

2t1

ia , A

Time,  s

Fig.  2.   Armature current response.

a1=A1A0−K1A2A02−K1A1, a2=−A1+a1A0K1,b1=a1−A0K1   (36)

D.  Parametric Identification

After having deduced the mathematical forms which are used  for calculation of the transfer function coefficients and which enable us at the same time to calculate the electric and mechanical motor parameters, we present here, the stages to be followed at the time of the determination of these parameters.

the calculation of Ki and Kv gains of the tow outputs ia(t) and ω(t) respectively, by taking account equations (8) and (9) gives

Ki=Δia=fΔUaK2+Raf                          (37)

Kv=Δω=KΔUaK2+Raf                         (38)

according to (37) and (38) we deduce f and μ

f=KΔiaΔω                                  (39)

μ=RafK2+Raf                                 (40)

by identification of H1(s) and H2(s) denominators with H(s) denominator we obtain

a1=τm+μτe                               (41)

a2=τmτe                                  (42)

according to (41) and (42) we can obtain a second order equation

μτe2−a1τe+a2=0                                  (43)

The resolution of the equation (43) gives two roots one is positive, the other is negative (rejected).

- According to (41) and (42) we deduce τm

- The deduction of τm and τe gives  La and J.

- The static torque can be calculated from steady state as

Tst=Kia0−fω0                                      (44)

Δω

ω0

ω1

1

Time,  s

Speed, rad/sec

Fig.  1.   Rotor speed angular response

- EXPERIMENTAL RESULTS

The separately excited dc motor used for experimental tests, has the nominal characteristics shown on table IV.  the first experiment is the determination of electric dc motor   parameters according to direct tests, like armature resistance Ra and armature inductance La, as well as the back-EMF constant K. Mechanical parameters are also determined by direct tests ( static torque Tst  and  viscous friction coefficient f ). The deceleration test, enables us to determine the moment of inertia J. The second experiment to be carried out is to identify the dc motor parameters from the dynamic test. According to a step amplitude ΔUa of terminal voltage applied to the armature circuit of the dc motor. The initial and final values of the armature current and the angular speed obtained from this test, are shown on  table III. The recording of ia(t) gives t1=0.026s where the current pass by its maximum. For the two identification methods  ( developed Pasek’s and moments methods), the back-EMF constant K, armature resistance Ra, and static torque Tst  can

A0A1A2a1a2b1ω(t)6.1218180.14034940-0.000940.0556000.0014400.012524ia(t)-0.29657-0.152533-0.0005040.4705650.240323-5.962602

TABLE I

1,2 AND 3 ORDER MOMENTS AND TRANSFER FUNCTION COEFFICIENTS VALUES

TABLE II

COMPARISON BETWEEN DIRECT TESTS, MOMENTS  AND PASEK’S DEVELOPED METHODS FOR PARAMETER IDENTIFICATION OF DC MOTOR

Ra

(Ω)La

(H)K

(N.m.A-1)J

(Kg.m2 )f

(N.m.s/rad)Tst

(N.m)Direct tests280.8201.340.00280.000540.127Moments method30.90.8031.3230.00310.00050.128Pasek’s developed method30.90.4381.3230.00360.00050.128

be determined by using respectively equations (5), (7) and (20). the viscous friction coefficient f, can be determined by using equation (19) for the first method (developed Pasek’s method) and by using (39) for the second (moments method). By using the first method and let us know Δia(t1) and Δia(2t1) , δ is then calculated from equation (16),( δ=0.79). from the abacus of  Fig. 7  we  deduce  respectively λ  = 4.5 and  t1/ τe =1.82. let us know  λ and τe we deduce τm and consequently J and La.

Table I shows the first, second and third order moments values ,as well as the transfer function coefficients values successively calculated from the trapezoids method. Let us know a1, a2 and μ  then, we have the second order equation 0.00874τe2−0.0556τe+0.00144=0, the resolution of this equation gives τe1=0.026s and τe2=6.34s( τe2 is a rather large time constant, is thus rejected). The deduction of τe and τm enables to calculate J and La. Table II summarizes the values of the parameters calculated from the two identification methods (direct tests, moments and Pasek’s developed methods).

Finally to check the precision of each method, we have simulated the dynamic test applying a step amplitude of terminal voltage ΔUa=188V, to the armature circuit of dc motor, as well as, deceleration test and mechanical characteristic.

According to Figs. 3, 4 and 5  the curves simulated from the dynamic test parameters with the moment method are close to the real curve (experimental curve) that those simulated from the direct tests parameters and Pasek’s developed method. With regard to the steady state (Fig.6) the curves simulated from the dynamic test and direct tests parameters are almost identical and close to real measurement (experimental curve).

Ua(V)ia(A)ω(rpm)Initial regime600.113400Final regime2480.1671745

TABLE III

DYNAMIC TEST

Time,  s

ia , A

Direct tests

Moments method

Experimental curve

Pasek’s developed method

Fig.  3.   Armature current response.

Time,  s

Speed, rad/sec

Direct tests

Moments method

Experimental curve

Pasek’s developed method

Fig.  4.   Rotor speed angular response

Time,  s

Speed, rad/sec

Fig.  5.   Deceleration  test

Direct tests

Moments method

Experimental curve

Pasek’s developed method

Speed, rad/sec

Torque, N.m

Fig.  6.   Mechanical characteristic

Direct tests

Moments method

Experimental curve

Pasek’s developed method

Fig.  7.   Abacuses for the determination  of  DC motor parameters .

- CONCLUSION

In this work, we tried to contribute our share in the discipline of the dc motors modeling. This contribution, which can be classified in a very wide field of identification methods can be summarized in the following results: we have developed a dynamic model based on the moments and Pasek’s methods. The moments method especially makes it possible to have a model closer to reality in transitory mode. We have proposed a comparison between various models based on the identification methods(direct tests, moments and Pasek’s methods), this comparison is made on the basis of real measurements taken in laboratory on a separately excited dc motor, with 180 W of rated power. It shows the advantage of the only dynamic test for identification, coupled to the moments method

Rated power180  WRated speed1500  rpmArmature voltage270  VField voltage220  VArmature current 1.1  AField current0.4  A

TABLE IV

SPECIFICATION OF EXPERIMENTAL DC MOTOR

APPENDIX

The constants L1, L2, L3 and L4 are given as

L1=1+a−2μ1−a+2μ−1a1+a−2μ1−a+2μ−1+a−2μ1−a+2μ−1

L2=12a1+a−2μ1−a+2μ−1a1−a1+a−2μ1−a+2μ−1−1+a1+a−2μ1−a+2μ

L3=1+a−2μ1−a+2μ−12a1+a−2μ1−a+2μ12−1+a−2μ1−a+2μ−12

L4=12a1+a−2μ1−a+2μ−12a1−a1+a−2μ1−a+2μ−12−1+a1+a−2μ1−a+2μ12

# REFERENCES

[1]   A. Rubaai, R. Kotaru , “Online identification and control of a dc motor using learning adaptation of neural networks,’’ IEEE Trans. Ind. Aplicat., vol. 36, pp. 935-942, May/June 2000.

[2]   J.C. Basilio and M.V. Moreira , “state-space parameter identification in a second control laboratory,’’ IEEE Trans. on Education , vol. 47, pp. 204-210,  May 2004.

[3]  O.  Touhami,  H. Guesbaoui, C. Iung, “Asynchronous machine   parameter identification using the recursive least-squares method ,’’ in Proc. ICEM, conf., pp. 458-462, Paris, sep. 1994.

[4]    S. Weerasooriya, M.A. 	 El-Sharkawi, “Identification and control of a dc motor using back propagation neural networks,’’IEEE Trans. Energy Conversion, vol. 36, pp. 663-669,  1991.

[5]  J.P. Louis, B. Multon, M. Lavabre, “Commande des machines à courant continu à vitesse variable,’’ Traité de Génie Electrique, D3610-3611, Techniques de l’ Ingénieur, Paris,  1998.

[6]    P. Coirault, J.C. Trigeassou, J.P. Gaubert, G. Champenois, “Parameter  estimation of an induction machine using reinitialized partial moment,’’ in Proc. CCA, conf. IEEE, pp. 979-984, Albany, 1995.

[7]  E. Etien, J.D. Gabano, L. Rambault, J. C. Trigeassou, D. Mehdi,  “Frequential moments: Application to controller reduction,’’ in Proc. ACC, conf., pp. 2869-2873 ,Chicago, Jun. 2000.

[8]  E. pasek, “Novy zpusob urcent zakaladnich dynamickych parametru stejnosmernero motoru,’’ elektrotech obz., vol. 51, pp. 109-114, 1962.